Government of Nepal

Vyas Municipality

Office of the Municipal Executive

Damauli, Tanahun

Baireni Land Pooling Project (First Phase)

Project Implementation Unit, Byas–10, Baireni

LAND CONTRIBUTION AND LAND RETURN POLICY

(Draft for Approval)

2079 BS (2022 AD)

# Table of Contents

Table of Contents	2

1.  Background	3

2.  Land Contribution and Land Return Policy	3

2.1  Preliminary	3

2.2  Land Contribution Policy	4

2.3  Land Return Policy	5

Schedule 1 — Land Contribution Percentage Tables	7

Table 1 — Contribution payable by every landowner for roads and common facilities	7

Table 2(a) — Additional contribution for special facilities	8

Table 2(b) — Additional contribution for two-frontage corner plots	8

Schedule 2 — Examples of the Land Contribution Policy	8

Approval	9

# 1.  Background

Land pooling (land readjustment) is defined as: “the work of forming a single parcel from land held by various individuals and institutions — with each landowner contributing land in proportion to their active participation and the benefits received — then providing physical infrastructure such as roads, drains, open spaces and other community facilities, re-dividing the area into regular, developed plots, and returning those plots to the original landowners.”

Under this concept, the Baireni Land Pooling Project is an effort initiated in Ward No. 10 of Byas Municipality, Tanahun, for the development of a planned residential area. Following a feasibility study, the first phase — covering approximately 261 ropani — was published in the Nepal Gazette by the Ministry of Urban Development on 2078/11/02, marking the start of project implementation.

Because the proposed infrastructure (roads, drains, open spaces and other community facilities) is to be funded from local resources without external investment, building-plot land of value equal to the construction cost must be deducted proportionally from the project-area landowners on the basis of the services and benefits they receive. A certain percentage of each parcel is deducted as a land contribution, and a portion of the contributed land is sold to meet the total project cost. The project therefore rests on two principal pillars:

- Infrastructure construction; and

- Land management — returning developed plots to every original landowner without exception, which is the more challenging task.

Preparing the re-adjustment (re-plotting) map and the long sheet therefore requires particular attention to three elements: the land return policy, the land contribution percentages, and the land contribution table (long sheet). This document sets out those policies clearly and transparently so that the detailed contribution due from each landowner can be computed and the re-plotting map prepared.

# 2.  Land Contribution and Land Return Policy

This Land Contribution and Land Return Policy has been prepared for the first-phase Baireni Land Pooling Project, taking into account the experience of land pooling projects previously implemented in Nepal and the suggestions obtained through detailed consultation with the consumer committee during implementation, in the interest of both the project and its consumers. It has been approved by the Project Consumer Committee and the Project Management Sub-Committee.

## 2.1  Preliminary

2.1.1 This policy shall be called the “Baireni Land Pooling Project Land Contribution and Land Return Policy.”

2.1.2 This policy shall come into force upon the decision of the Project Consumer Committee and the Project Management Sub-Committee.

2.1.3 This policy shall apply throughout the project implementation period and thereafter.

## 2.2  Land Contribution Policy

2.2.1 To construct the infrastructure (roads, drains and the like) and to provide the other services required to operate the project, land shall be deducted as a contribution from each landowner within the project area, on the proportion of the benefits and facilities that landowner receives.

2.2.2 Every landowner shall contribute land for roads and drains according to the proposed road width. For parcels that abut an existing road, the land falling within the depth of the first parcel of the proposed block shall be deducted as a contribution, on the basis of the proposed road width and the existing road width, in accordance with the table in Schedule 1, Table 1.

2.2.3 Where the road width recorded in the field differs from that on the survey map, or where a new road is opened, the contribution shall be calculated and deducted from the parcel on the basis of the survey map.

2.2.4 For internal branch (khande) roads: where the majority of the relevant landowners request the road as it exists in the field, half of the area of the additional field road width shall be deducted from the parcels of the relevant landowners, and the remaining required contribution shall be deducted from the rest.

2.2.5 Where the ownership certificate records a road in the sole ownership of one landowner but the road has, by mutual understanding, been used collectively: upon submission of an application together with all documents evidencing that mutual understanding, the land required as a contribution for the road shall be apportioned arithmetically so that all relevant landowners bear it.

2.2.6 When new parcels are carved, for a parcel fronting two roads both frontages shall be retained, and the land required for both roads shall be deducted with the help of the contribution table in Schedule 1, Table 1.

2.2.7 When new parcels are carved, for a parcel at the corner of two roads the frontage shall be set toward the larger road and the land required for the larger road deducted in accordance with Schedule 1, Table 1; in addition, for each such two-frontage parcel an additional land contribution shall be deducted in accordance with Schedule 1, Table 2(b).

2.2.8 For the land required for all road junctions and corner turnings, an additional contribution shall be deducted from each developed plot at the percentage fixed in the contribution table in Schedule 1, Table 1.

2.2.9 For land adjoining an open space, and where a road lies on the boundary of the open space, an additional contribution shall be deducted from the land in front of the open space at the percentage fixed in Schedule 1, Table 2(a). In this policy, “land adjoining an open space” means only land within 8 m frontage of the open-space boundary, and “land in front of the open space” means only land lying directly across the road from the open-space frontage. This is clarified by the example in Schedule 2.

2.2.10 For the original sloped parcels on the northern boundary of the project (the Boteni Kholsi side), because infrastructure must be built to develop plots on that sloped land, an additional contribution shall be deducted at the percentage fixed in Schedule 1, Table 2(a). Likewise, for original parcels falling in the depression (dhaap) in the middle of the project area, an additional contribution shall be deducted in accordance with Schedule 1, Table 2(a).

2.2.11 Opening a road from the project’s developed plots to serve land that lies outside the project but adjoins its boundary shall not be permitted, as this would cause a future shortage of roads and drains.

## 2.3  Land Return Policy

2.3.1 Any parcel shall generally be returned at its former location or nearby, in accordance with the decision of the Project Consumer Committee. (Where a road, open space, or community or public-interest area comes to occupy the former location, the parcel shall, as far as possible, be returned at a nearby location.)

2.3.2 The minimum parcel area under the proposed land-use classification — 130 sq. m (0-4-0-1.42 ropani) — shall be maintained. However, for parcels whose developed area after contribution is less than 130 sq. m and which were already smaller before subdivision was frozen in the project area, parcels smaller than the minimum may be returned. The absolute minimum developed-plot area shall be 79.50 sq. m (0-2-2-0 ropani).

2.3.3 The minimum frontage under the proposed land-use classification shall be maintained. The minimum frontage of a developed plot shall be 6.0 m.

2.3.4 For parcels abutting former main roads, and for similar parcels that would otherwise be adversely affected, the project shall not be bound to meet the stated minimum frontage and area (only where it is technically not feasible to apply policies 2.3.2 and 2.3.3).

2.3.5 In the land-return process, the amount of any transaction (purchase or sale) between the project and a landowner shall be fixed at the price set by the project on the basis of the prevailing sale rates around the project area.

2.3.6 Where a developed plot to be returned would be smaller than 79.50 sq. m, the project shall add the shortfall land to bring it to the minimum plot area and return it to the owner; the owner shall purchase the added land from the project at the price fixed under policy 2.3.5.

2.3.7 If the owner is unwilling or unable to take the added land and the prescribed developed plot, the project may pay the owner the price fixed under policy 2.3.5 and purchase such land.

2.3.8 Where land must be added to reach the minimum area and the owner, citing inability to pay a lump sum, so requests, and the Consumer Committee finds the request valid, the Project Management Sub-Committee may, by unanimous decision, forward a recommendation. On the basis of that recommendation, possession may be granted on condition of payment within the period estimated for completing the remaining project work, and the ownership certificate shall be provided after payment is made in full.

2.3.9 Where a parcel to be returned would be smaller than 79.50 sq. m or would have a frontage of less than 6 m, two or more landowners may jointly take a single parcel.

2.3.10 Generally, a single landowner holding up to 200 sq. m (0-6-1-0.64) in total, or one household’s several parcels at different locations, may be consolidated and returned at one location.

Example: where one person holds land at three places — 60 sq. m, 150 sq. m and 250 sq. m — the 60 sq. m and 150 sq. m are returned together as one parcel and the 250 sq. m as a separate parcel. If requested, parcels larger than 150 sq. m may also be returned together at one place, provided the adjacent return-location parcel does not object and it is technically feasible.

2.3.11 When a new parcel is carved on land containing a house, a setback of at least 1 m on the window/door side and 1 m on the road side shall, as far as possible, be kept. If the newly created parcel exceeds the owner’s entitlement, the additional land shall be returned at the price fixed under policy 2.3.5.

2.3.12 The depth of the various blocks holding developed plots shall generally be 26 m to 40 m — that is, a parcel depth of 13 m to 20 m. (Maintaining the minimum developed-plot area of 79.50 sq. m and the minimum frontage of 6 m yields a minimum parcel depth of 13.5 m.)

2.3.13 When new parcels are carved at a corner, first priority shall be given to the parcel already at the corner. Where the original owner does not wish to take the proposed developed corner plot, it shall be given to a nearby larger holding. If a larger holding cannot be accommodated there when the new plot is prepared, it shall be kept as a Sales Plot.

2.3.14 For a corner parcel where the area remaining after contribution is less than the minimum: if the owner wishes to take land at that same corner, the shortfall shall be purchased from the project at the price fixed under policy 2.3.5. If the owner does not wish to take the proposed corner parcel, it may — where it is feasible to reach the minimum area — be returned at a nearby location rather than at the corner.

2.3.15 Where two or more roads are proposed through a single original plot so that it would be split into more than one corner plot, generally only one parcel shall be returned, at the corner with the largest area.

2.3.16 After project completion, developed plots shall not be subdivided below the area and frontage fixed by the prevailing law of the Government of Nepal.

# Schedule 1 — Land Contribution Percentage Tables

## Table 1 — Contribution payable by every landowner for roads and common facilities

The figures below give the total contribution percentage for each combination of existing road width and proposed road width, by block-depth band. Each total already includes the fixed common-facility shares — open area 5.04%, infrastructure 4.79%, and road corner 1.00% (10.83% combined) — added to the geometric road share. The returned parcel depth equals half the proposed block depth.

| Existing road width (m) | Block depth (m) | 4 m | 6 m | 7 m | 8 m | 9.14 m |
| --- | --- | --- | --- | --- | --- | --- |
| Block depth band 26–28 m (proposed block depth 27 m; returned parcel depth 13.5 m) |  |  |  |  |  |  |
| 0 (no road) | 27 | 23.73% | 29.01% | 31.42% | 33.69% | 36.12% |
| 4 | 27 | 10.83% | 17.73% | 20.83% | 23.73% | 26.82% |
| 6 | 27 | 2.83% | 10.83% | 14.40% | 17.73% | 21.25% |
| 9.14 | 27 | −12.68% | −2.33% | 2.22% | 6.42% | 10.83% |
| Block depth band 28–30 m (proposed block depth 29 m; returned parcel depth 14.5 m) |  |  |  |  |  |  |
| 0 (no road) | 29 | 22.95% | 27.97% | 30.27% | 32.45% | 34.79% |
| 4 | 29 | 10.83% | 17.28% | 20.21% | 22.95% | 25.89% |
| 6 | 29 | 3.42% | 10.83% | 14.16% | 17.28% | 20.60% |
| 9.14 | 29 | −10.71% | −1.31% | 2.86% | 6.74% | 10.83% |
| Block depth band 30–32 m (proposed block depth 31 m; returned parcel depth 15.5 m) |  |  |  |  |  |  |
| 0 (no road) | 31 | 22.26% | 27.05% | 29.25% | 31.34% | 33.60% |
| 4 | 31 | 10.83% | 16.89% | 19.65% | 22.26% | 25.05% |
| 6 | 31 | 3.93% | 10.83% | 13.96% | 16.89% | 20.03% |
| 9.14 | 31 | −9.05% | −0.44% | 3.41% | 7.01% | 10.83% |
| Block depth band 32–34 m (proposed block depth 33 m; returned parcel depth 16.5 m) |  |  |  |  |  |  |
| 0 (no road) | 33 | 21.64% | 26.21% | 28.33% | 30.34% | 32.52% |
| 4 | 33 | 10.83% | 16.54% | 19.16% | 21.64% | 24.31% |
| 6 | 33 | 4.38% | 10.83% | 13.77% | 16.54% | 19.52% |
| 9.14 | 33 | −7.62% | 0.31% | 3.90% | 7.25% | 10.83% |
| Block depth band 34–36 m (proposed block depth 35 m; returned parcel depth 17.5 m) |  |  |  |  |  |  |
| 0 (no road) | 35 | 21.09% | 25.46% | 27.50% | 29.43% | 31.54% |
| 4 | 35 | 10.83% | 16.24% | 18.72% | 21.09% | 23.64% |
| 6 | 35 | 4.77% | 10.83% | 13.61% | 16.24% | 19.06% |
| 9.14 | 35 | −6.38% | 0.97% | 4.32% | 7.46% | 10.83% |
| Block depth band 36–38 m (proposed block depth 37 m; returned parcel depth 18.5 m) |  |  |  |  |  |  |
| 0 (no road) | 37 | 20.59% | 24.78% | 26.74% | 28.61% | 30.64% |
| 4 | 37 | 10.83% | 15.96% | 18.33% | 20.59% | 23.03% |
| 6 | 37 | 5.12% | 10.83% | 13.46% | 15.96% | 18.65% |
| 9.14 | 37 | −5.30% | 1.56% | 4.69% | 7.65% | 10.83% |
| Block depth band 38–40 m (proposed block depth 39 m; returned parcel depth 19.5 m) |  |  |  |  |  |  |
| 0 (no road) | 39 | 20.13% | 24.16% | 26.05% | 27.85% | 29.82% |
| 4 | 39 | 10.83% | 15.71% | 17.97% | 20.13% | 22.47% |
| 6 | 39 | 5.42% | 10.83% | 13.33% | 15.71% | 18.28% |
| 9.14 | 39 | −4.35% | 2.07% | 5.02% | 7.82% | 10.83% |

Note: A negative value (shown in red) means the existing road is wider than the proposed road, so the landowner regains land rather than contributing it. The road share is computed geometrically as ((W − E) ÷ 2) ÷ ((block depth ÷ 2) + (W − E) ÷ 2), where W is the proposed and E the existing road width.

## Table 2(a) — Additional contribution for special facilities

| S.N. | Parcel description | Additional contribution |
| --- | --- | --- |
| 1 | Land adjoining an open space (within 8 m frontage of the open-space boundary) | 3.00% |
| 2 | Land directly across the road from an open space | 2.00% |
| 3 | Land falling in the central depression (dhaap) of the project area | 8.00% |
| 4 | Original land with slope steeper than 30 degrees | 12.00% |
| 5 | Corner plot with two frontages | As per Table 2(b) |

Note: These additional contributions do not apply to all plots — only to parcels meeting the stated condition.

## Table 2(b) — Additional contribution for two-frontage corner plots

| S.N. | Corner type (main road × branch road) | Additional contribution |
| --- | --- | --- |
| 1 | 9.14 m × 8 m | 9.00% (maximum) |
| 2 | 8 m × 8 m | 8.61% |
| 3 | 8 m × 7 m | 8.23% |
| 4 | 8 m × 6 m | 7.90% |
| 5 | 7 m × 7 m | 7.29% |
| 6 | 7 m × 6 m | 6.98% |
| 7 | 6 m × 6 m | 6.68% |
| 8 | 7 m × 4 m | 4.74% |
| 9 | 6 m × 4 m | 4.23% |
| 10 | 4 m × 4 m | 4.00% (minimum) |

Note: The additional corner contribution ranges from a minimum of 4.00% to a maximum of 9.00%. The applicable value is selected by the main-road and branch-road widths at the corner. These row values should be confirmed against the officially signed Schedule before production use.

# Schedule 2 — Examples of the Land Contribution Policy

This Schedule illustrates the application of the contribution policy with worked examples and diagrams — in particular the meaning of “land adjoining an open space” and “land in front of an open space” (policy 2.2.9), corner and two-frontage parcels (policies 2.2.6 and 2.2.7), and the treatment of sloped and depression land (policy 2.2.10). The illustrative figures and worked calculations are to be attached here.

# Approval

This policy is hereby approved by the Project Consumer Committee and the Project Management Sub-Committee of the Baireni Land Pooling Project (First Phase).

…………………………	…………………………

Chairperson, Project Consumer Committee	Coordinator, Project Management Sub-Committee

…………………………	…………………………

Date: ………………	Date: ………………