RePlot — Baireni Land Readjustment Tool

Policy Parameters Reference

Land Contribution & Return — Application Variable Catalog

This document lists every parameter and variable the application reads or writes, grouped into four buckets: policy configuration, per-parcel inputs, computed outputs, and replotting variables.

Companion to: baireni-lpp-phase1.policy.json · POLICY-SPEC.md · PARAMETERS-AND-INTEGRATION.md

# Contents

Contents	2

How to read this document	3

Bucket 1 — Policy parameters (configuration)	3

1.1  Always-on contribution rates	3

1.2  Road-contribution formula	3

1.3  Situational additional rates (Schedule 1, Table 2a)	3

1.4  Corner-plot table (Schedule 1, Table 2b)	4

1.5  Return constraints (Section 2.3)	4

1.6  Policy metadata	4

Bucket 2 — Per-parcel inputs	6

Bucket 3 — Computed outputs	6

Bucket 4 — Replotting & ledger variables	6

4.1  Per-allocation variables	7

4.2  Per-owner ledger (roll-up)	7

Notes	7

# How to read this document

Variables flow one direction through four buckets, and each bucket has a single owner. Configuration is set once per project; inputs are captured or derived per parcel; outputs are produced by the calculation engine; replotting variables track allocation against the computed targets.

In the input tables, sources shown in red are geometry-derived — computed by the NetTopologySuite layer from the drawn map. The calculation engine never sees geometry, only these derived values. Values shown as “(project sets)” have no fixed default and are entered per project.

Default values shown are those for the Baireni Land Pooling Project (First Phase). Because the policy is configurable, another project may carry different values for the same parameters.

# Bucket 1 — Policy parameters (configuration)

Set once per project and stored, versioned, inside the .lpp file. These are the only values that change from project to project.

## 1.1  Always-on contribution rates

| Parameter | Type | Value / Default | Description |
| --- | --- | --- | --- |
| openAreaRate | double | 0.0504 | Open-area share deducted from every parcel. |
| infrastructureRate | double | 0.0479 | Infrastructure construction share, every parcel. |
| roadCornerRate | double | 0.0100 | Road corner / turning share, every parcel. |
| fixedCommonTotal | double | 0.1083 | Sum of the three rates above; convenience constant. |

## 1.2  Road-contribution formula

Road share is computed geometrically as ((W − E) ÷ 2) ÷ ((D ÷ 2) + (W − E) ÷ 2), where W = proposed road width, E = existing road width, D = block depth. The result is signed: a negative value means the owner regains land.

| Parameter | Type | Value / Default | Description |
| --- | --- | --- | --- |
| roadFormulaId | string | "RoadContributionStandard" | Selects the registered road-contribution formula. |
| returnedParcelDepthRatio | double | 0.5 | Returned parcel depth = block depth × this ratio. |

## 1.3  Situational additional rates (Schedule 1, Table 2a)

| Parameter | Type | Value / Default | Description |
| --- | --- | --- | --- |
| openSpaceAdjoiningRate | double | 0.03 | Added when parcel adjoins an open space. |
| openSpaceAcrossRoadRate | double | 0.02 | Added when parcel is across the road from open space. |
| centralDepressionRate | double | 0.08 | Added for parcels in the central depression (dhaap). |
| steepSlopeRate | double | 0.12 | Added for slope above the threshold. |
| steepSlopeThresholdDegrees | double | 30 | Slope (degrees) above which the steep-slope rate applies. |
| openSpaceAdjoiningFrontageMaxM | double | 8 | Frontage band (m) that qualifies as “adjoining”. |

## 1.4  Corner-plot table (Schedule 1, Table 2b)

| Parameter | Type | Value / Default | Description |
| --- | --- | --- | --- |
| cornerContributionTable | CornerRow[] | 10 rows | Lookup of {main, branch, rate} by corner road widths. |
| cornerRateMin | double | 0.04 | Lower clamp on the corner rate. |
| cornerRateMax | double | 0.09 | Upper clamp on the corner rate. |
| cornerMatchToleranceM | double | 0.25 | Tolerance (m) when matching road widths to a table row. |

## 1.5  Return constraints (Section 2.3)

| Parameter | Type | Value / Default | Description |
| --- | --- | --- | --- |
| minPlotAreaSqM | double | 79.5 | Absolute minimum developed-plot area. |
| preferredMinPlotAreaSqM | double | 130 | Preferred minimum per land-use class. |
| minFrontageM | double | 6 | Minimum frontage of a developed plot. |
| minDepthM | double | 13 | Minimum parcel depth. |
| maxDepthM | double | 20 | Maximum parcel depth. |
| blockDepthMinM | double | 26 | Minimum block depth. |
| blockDepthMaxM | double | 40 | Maximum block depth. |
| setbackRoadM | double | 1 | Road-side setback when a house is present. |
| setbackWindowM | double | 1 | Window/door-side setback when a house is present. |
| consolidationMaxAreaSqM | double | 200 | Scattered holdings up to this may merge to one location. |
| jointReturnThresholdSqM | double | 150 | Above this, merged return only on request. |
| jointReturnAllowedBelowMin | bool | true | Two+ owners may share one sub-minimum parcel. |
| salesPlotForUnfittedCorners | bool | true | An unfittable corner becomes a sales plot. |
| projectLandRatePerSqM | double | (project sets) | Price for top-up purchase or buy-out. |

## 1.6  Policy metadata

| Parameter | Type | Value / Default | Description |
| --- | --- | --- | --- |
| policyId | string | — | Stable identifier for the policy. |
| policyVersion | string | — | Version stamp; copied onto every result. |
| status | enum | draft / approved | Draft policies are editable; approved ones are locked. |
| effectiveFrom | date | — | Date the policy takes effect. |

# Bucket 2 — Per-parcel inputs

Captured or derived for each original (sabik) parcel before the engine runs. Red sources are geometry-derived by NetTopologySuite; all others are entered or imported.

| Field | Type | Source | Feeds / used by |
| --- | --- | --- | --- |
| parcelId / kittaNo | string | Manual / Excel / DXF | Parcel identity. |
| ownerIds | string[] | Manual / Excel | Ledger, joint-return handling. |
| originalAreaSqM | double | Excel / geometry | Entitlement base. |
| frontageM | double | Geometry (NTS) | Return validation. |
| existingRoadWidthM (E) | double | Manual / survey map | Road formula. |
| proposedRoadWidthM (W) | double | Geometry-derived | Road formula. |
| blockDepthM (D) | double | Geometry-derived | Road formula. |
| isCorner | bool | Geometry-derived | Corner component; return priority. |
| cornerMainRoadM | double | Geometry-derived | Corner table lookup. |
| cornerBranchRoadM | double | Geometry-derived | Corner table lookup. |
| frontageCount | int | Geometry-derived | Two-frontage handling. |
| openSpaceRelation | enum | Geometry-derived | Table 2(a) situational rate. |
| inCentralDepression | bool | Geometry-derived / manual | Table 2(a) situational rate. |
| slopeDegrees | double | DEM / manual | Table 2(a) steep-slope rate. |
| touchesExistingRoad | bool | Geometry / survey | Clause 2.2.2 applicability. |
| branchRoadMajorityRequested | bool | Manual flag | Special rule 2.2.4. |
| hasCollectiveRoadAgreement | bool | Manual flag | Special rule 2.2.5. |
| hasHouse | bool | Manual / survey | Setback rule 2.3.11. |

# Bucket 3 — Computed outputs

Produced by the calculation engine per parcel. The full breakdown is persisted, not just the total — the line items are what make a contribution auditable and explainable to each owner.

| Field | Type | Meaning |
| --- | --- | --- |
| lines | ContributionLine[] | Each applied component: {id, name, ratePct, applied}. |
| totalContributionRate | double (signed) | Sum of applied component rates. |
| contributionAreaSqM | double | originalArea × total. |
| entitlementAreaSqM | double | originalArea × (1 − total) — the replot target. |
| policyVersion | string | Stamp copied from the policy for reproducibility. |

# Bucket 4 — Replotting & ledger variables

Created as the user draws developed parcels on the canvas. The entitlement area from Bucket 3 is the target; these variables track how each allocation compares to it.

## 4.1  Per-allocation variables

| Field | Type | Meaning |
| --- | --- | --- |
| newParcelId | string | Developed-plot identifier. |
| assignedOwnerId | string | Owner receiving the plot. |
| allocatedAreaSqM | double | Area as drawn (from geometry). |
| allocatedFrontageM | double | Frontage as drawn. |
| allocatedDepthM | double | Depth as drawn. |
| deltaToEntitlementSqM | double | allocated − entitlement (over / under). |
| flags | ReturnFlag | BelowMinArea, BelowMinFrontage, BelowMinDepth, NeedsTopUpPurchase, NeedsJointReturn, IsSalesPlot, CornerPriority. |
| topUpAreaSqM | double | Shortfall the project adds to reach the minimum. |
| topUpAmount | double | topUpArea × projectLandRatePerSqM. |

## 4.2  Per-owner ledger (roll-up)

| Field | Type | Meaning |
| --- | --- | --- |
| totalEntitlementSqM | double | Sum of the owner’s entitlements. |
| totalAllocatedSqM | double | Sum of areas actually allocated. |
| balanceSqM | double | entitlement − allocated. |
| purchaseDue | double | Amount owed by the owner for top-up land. |
| refundDue | double | Amount owed to the owner for under-allocation. |

# Notes

Corner-table row values should be confirmed against the officially signed Schedule before production use.

A few clauses are not simple additive constants — branch-road half-width (2.2.4), collective-use roads (2.2.5), setbacks (2.3.11) and corner priority / sales plots (2.3.13–2.3.15). These are handled by dedicated logic in the geometry and replotting layers rather than as contribution components, so the core additive engine stays clean.

The Baireni source policy is a draft; parameter values become final when the policy status is set to approved and the version is locked.